NAME: Tanmay Karmarkar                               ROLL NO: 21143

CLASS: S.E. COMP                                                 SUBJECT: DEL

EXPT. NO. 9                                                          DATE: 16/12/2021

TITLE: REALISATION OF MOD-N COUNTER USING DECADE COUNTER IC-7490

OBJECTIVES:

1. To verify IC-7490

2. To design and implement MOD-6

3. To design and implement MOD-96

APPARATUS:

Digital trainer kit, IC 7490, Patch cord, +5V Power supply.

THEORY:

IC 7490 is a TTL MSI decade counter. It contains four master slave flip flops and a few logic gates to provide a divide by two counter and a 3 stage binary counter which provides a divide by 5 counter.

IC 7490 is a 14 pin IC.

Pin Diagram:

Simplified Internal Diagram of IC 7490:

- If both the reset input R0(1) and R0(2) are at logic 1 then all the flip flops will be reset and the output is given by-

QdQcQbQa=0000.

- If both the reset input Ra(1) and Ra(2) are at logic 1 then the counter output is set to decimal 9.

QdQcQbQa=1001.

- If any one pin of R0(1) and R0(2) and one of Ra(1) and Ra(2) are at low, then the counter will be in counting mode.

Reset/Count Truth Table:

| Reset Inputs |  |  |  | Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R0(1) | R0(2) | Ra(1) | Ra(2) | Qd | Qc | Qb | Qa |
| 1 | 1 | 0 | X | 0 | 0 | 0 | 0 |
| 1 | 1 | X | 0 | 0 | 0 | 0 | 0 |
| X | X | 1 | 1 | 1 | 0 | 0 | 1 |
| X | 0 | X | 0 | COUNTER |  |  |  |
| 0 | X | 0 | X | COUNTER |  |  |  |
| 0 | X | X | 0 | COUNTER |  |  |  |
| X | 0 | 0 | X | COUNTER |  |  |  |

Logic Diagram:

Truth Table:

| Count | Qd | Qc | Qb | Qa |
| --- | --- | --- | --- | --- |
| 0(start) | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 |
| 6 | 0 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 |
| 10(new cycle) | 0 | 0 | 0 | 0 |

Design and implementation of MOD-6 Counter:

Mod-6 counter counts through 6 states.

Truth Table:

| Qd | Qc | Qb | Qa | Y |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1(invalid) |
| 0 | 1 | 1 | 1 | 1(invalid) |
| 1 | 0 | 0 | 0 | 1(invalid) |
| 1 | 0 | 0 | 1 | 1(invalid) |

K-Map for Reset Logic:

Logic Diagram:

State Diagram:

From all the invalid states (6,7,8,9) and from 6. The counter should reset.

Design and implementation of MOD-96 counter:

We have to implement MOD-96 counter. So upto MOD-100 two IC7490s will be sufficient, i.e. we have to use two decade counters.

A MOD-96 counter counts through 96 states through 0 to 95. Therefore, the counter should reset as sson as the count becomes 96.

So in order to reset the counter at 96, connect the Q outputs, which are equal to 1 in the count of 96 to an AND gate and then connect the AND output to Ro(1) and Ro(2) i.e., reset inputs of both the ICs.

Reset Logic:

Logic Diagram:

Applications of IC 7490 (Decade counter):

1. It is used in digital counter circuits.

2. It is used in digital timers and clocks.

3. It is used in automatic controller circuits.

CONCLUSION:

Hence, we have verified IC 7490 and also have successfully designed and implemented MOD-6 counter and MOD-96 counter using IC 7490.

REFFRENCE:

- R.P.Jain “Modern Digital Electronics” TMH 4th Edition

- D.Leach,Malvino,Saha,”Digital Principles and Applications”,TMH

Subject teacher Sign with Date  	 	                   Remark